Here are some measurements of today's event (July 15, 2009):

Gauge            	Lat    	Long   	Arr time	Max time	Ampltude		Period

Spring Bay AU		42.5S	147.9E	1145Z		1220Z		0.06m /  0.2ft 		10min

Port Kembla AU		34.5S	150.9E	1200Z		1323Z		0.14m /  0.5ft 		12min

Jackson Bay NZ		44.0S	168.6E	0956Z		1155Z		0.50m /  1.6ft 		22min

DART 55015	  	46.9S	160.6E	0950Z		0953Z		0.05m /  0.2ft  	08min

Here are some monitor traces.

DTAS is DART 55015. Note that the tsunami does not arrive until 0950(the earlier stuff is the earthquake). The y-axis is water depth in meters with the tide removed.

For coastal tide gauges, the y-axis is departure from sea level (meters) with tide removed.

JBAY is Jackson Bay on the west coast of South Island.

PKEM is Port Kembla, New South Wales.

SPRG is Spring Bay, Tasmania.

Here are some more NZ measurements analysed for the 15 July Fiordland tsunami event:

Gauge            	Lat    	Long   	   Arr time	Max time      Amplitude	Period

Jackson Bay NZ      	43.96S	168.62E   0956Z		1155Z	        0.47 m      	18-28 min

Charleston NZ	       	41.91S	171.43E   1104Z		1117Z	        0.25 m      	13-15 min

Dog Island NZ	       	46.65S	168.41E   1106Z		1207Z	        0.12 m      	4-6 min initially then

12-17 min