INDONESIA REPORT

received from Dr. Prih Harjadi, Deputy Director, BMKG, 16 March 2011

The InaTEWS (Indonesia Tsunami warning System) has issued Tsunami Warning following the Erathquake generated tsunami in Japan last Friday 11 March 2011. The warning was issued for the eastern part of Indonesia including northern coast of Papua Indonesia (2 provinces), north Maluku province and north Sulawesi province. The Warning was very good responded by the local governments and the coastal communities at the said potentially struck by tsunami. Updated informations were issued based on the observations of tide gauges that were accessed at IOC as well as the indonesian agency responsible for tide gauge monitoring and broadcasted every 1 to 1,5 hours. The progress of the tsunami propagation was real time broadcasted by one national television.

The tsunami observed in tide gauges stations of Indonesia on 11.50 UTC at Bitung north Sulawesi 0,1 m and on 12.05 UTC at Jailolo north Maluku 0.1 m. The data of tide gauges at Jayapura (0.85 m ) Sarmi (0,93m) Raja Ampat (0.47m), Gebe (0.22m) in Papua were obtained later. Time arrival of tsunami around 12.00 UTC based on tsunami record analysis.

In Papua Indonesia, the tsunami was reported generated some damages at four district coastal areas consists of several houses destructed and some completely swept away, bridges damaged